- Why Systolic arrays?

In CPU, GPU doing the math w.x+b requires const. R/W into cache / memory, which is called Von Neumann Bottleneck.

To avoid that we use systolic arrays (in TPU architecture) which contains PE (Processing Elements), talk to neighboring elements only, reducing delay.

- Weights: "How important is this pixel?"

y = mx + b, m = slope / weight

neuron selects the pixel if m is greater else ignores.

- Biases:

b, the bias (y-intercept)

A bias is a threshold of confidence. If the bias is -50, the neuron is incredibly stubborn. The pixel weights have to sum up to at least +51 just to overcome the bias and make the neuron fire. It prevents the network from triggering on random noise.

- Matrices ??

Ex: if we have 16 neurons each one requiring 784 weights, (i.e., 12,544 total weights), usage of for-loops makes the CPU do calculations in a sequential manner (huge delay)

Matrix operations in a GPU does them in a simultaneous fashion in a single clk cycle using SIMD (Single Instruction Multiple Data).

Phase 1: Synthesis Results

| Resource | Used | Utilization |
| --- | --- | --- |
| LUT | 9,086 / 23,040 | 39.4% |
| Registers | 3,019 / 23,280 | ~13% |
| BSRAM | 0 / 56 | 0% |
| ALU | 4,063 | — |
| Logic combined | 13,151 / 23,040 | 58% |
| I/O | 3,109 | Very high |

64 PEs, 1MAC / clk @ 75.315 MHz

=> 64 x Fmax => 4.820 GMAC/s

=> 2 x 4.820 GMAC/s

=> 9.64 GOPS (Giga Operations Per Second)

Inference: No usage of BSRAM, and more usage of physical I/O ports

| Metric | Old Wallace 8×8 | New DSP MAC |
| --- | --- | --- |
| LUT | ~9,086 | 3,054 |
| Register | ~3,019 | 2,099 |
| Logic utilization | ~58% | 29% |
| DSP | 0 | 28 |
| BSRAM | 0 | 0 |
| Fmax | 75.315 MHz | 96.490 MHz (12.3 GOPS) |
| Worst slack | +6.723 ns | +9.636 ns |

Phase 2: Proposed Ping Pong architecture

Time ──────────────────────────────→

PING:  LOAD A1/B1   COMPUTE A1/B1   LOAD A3/B3   COMPUTE A3/B3

PONG:                LOAD A2/B2     COMPUTE A2/B2 LOAD A4/B4